Rational Method Homework

- Find the composite runoff coefficient, c, for a drainage basin having the following cover conditions:

| Cover | Drainage Area (square ft) | Drainage Area (acres) | C |
| --- | --- | --- | --- |
| Pavement | 2500 |  | 0.9 |
| Roofs | 2000 |  | 0.9 |
| Driveway | 800 |  | 0.9 |
| Grassed Areas |  | .45 | 0.3 |
| Woods |  | 1.21 | 0.2 |

Composite C = _____________________

- Compute the peak runoff for a 15-year storm using the rational method:

Area is 14.06 acres (Impervious-.06 acres; Grass-6.5 acres; Woods -7.5 acres)

Time of Concentration Data: (include TR-55 worksheet 3)

Overland Flow-average grass; 100’; 5% slope

Shallow Concentrated:  50’ long 7% slope

Use IDF curve you developed for Project 1 to determine the intensity.

Peak Flow (frequency of 15 years) = ______________________